TerEx Tool tutorial: An Arc Toolbox to semi-autonomously select and delineate terrace and floodplain features from lidar

Justin Stout

Utah State University

2012

Toolboxes are valid for Arc 10.0 and Arc 10.1

Terrace extraction tool: Tutorial

TerEx utilizes a high resolution DEM to select, map and measure discrete, near-channel flat surfaces (fluvial terraces and floodplains) based on local relief and several other user-specified inputs.  We have attempted to make the tool sufficiently flexible (based on user-inputs) to be used for delineation of both strath and fill terraces at either the watershed or reach scale.  It can either be run in a PYTHON environment or as a Toolbox in ArcDesktop (ESRI, Redlands, CA). For additional information on the tool, see: Stout and Belmont (2013) TerEx Toolbox for semi-automated selection of terrace and floodplain features from lidar.  ESPL.

By offering to the user many knobs to “turn” and ultimately autonomously select flat surfaces from a DEM the user can employ any geomorphic intuition when choosing and refining the available parameters.  The tool uses topographic analysis of the DEM to identify flat surfaces based on three attributes: 1) local relief of the surface; which cannot exceed a user defined change in elevation (local relief) over a focal window; 2) the area of the surface, once identified, must be greater than the user specified area (measured in square meters (m2)) and 3) the selected surface must be within the valley width (m) of the channel centerline. A flow chart of the tool (Figure 1) illustrates the how the inputs, processes and outputs are related.

Figure 1

To begin, as with any exercise in GIS, it is imperative that the user have the DEM and all input shapefiles defined to be in the same projection.  The units used in the input parameters must match the units of the projection of the input DEM and stream shapefile.  For example, if using the “UTM Zone 15 NAD83” projection the units of the input parameter must be in meters and square meters.  This tool, has been scripted in the Python language, but requires a “Spatial Analyst” extension along with ArcMap. The tool described above is available both in Python code and as a tool box which can be loaded into ArcMap. We have found the tool is most stable in Python (we have used Programmers Notepad (http://www.pnotepad.org/download/)). The tool can be downloaded from http://justinstout.webs.com/gisandtools.htm or from the supplemental material.

Tutorial:

This tutorial is to simply provide the user a very basic example to so that they can make sure the scripts were installed correctly, and to demonstrate the order of operations to execute the tool and offer an example of the use the products derived from the DEM.

Step 1: Download and save the zipfile to a location that is easily accessible.  The zipfile can be downloaded from http://justinstout.webs.com/gisandtools.htm

Step 2: Add toolboxes

If you are using Arc 10.0, install the toolboxes in ArcCatalog or if you are using Arc 10.1 the toolboxes can be added to ArcMap.  Due to the necessity of editing the output files and the fact that ArcMap often leaves lock files (.loc) which won’t allow the second step of the tool to run.  For this reason the tool will only run in Arc Catalog for Arc 10.0 users, however ESRI fixed the errors and the tool can be run from ArcMap in Arc 10.1. Alternatively if the user has sufficient exposure to the Python language, the tool can be run in a text compiler such as Programmers Notepad.   To install the toolboxes, first open an instance of Arc Catalog. Once Arc Catalog is open, open the toolbox and right click and choose Add Toolbox.  Browse to where you downloaded and extracted the zipfile.

Step 3:  Run Step1 of the tool

The user specifies the focal area widow over which local relief (range in elevation) is calculated. The cells of the newly calculated relief map are selected by a conditional statement selecting only the cells where the change in relief is greater than the change in elevation specified by the user, and is subsequently converted to polygon shapefile representing flat surfaces on the landscape. The areas of all the polygons are calculated; any polygon which is smaller than the user defined minimum terrace area is removed.  Surfaces which lie within the valley width are then selected, and based upon  how close to one another the polygons are (one cell’s distance) the multiple polygons are aggregated into single polygons. The edges of the aggregated polygons are then smoothed using the smoothing tolerance chosen by the user.  The smoothing tolerance dictates the generalized shape of the polygon. Smoothing tolerance is likely a function of channel sinuosity; meaning how much the river has dissected and cannibalized the river terrace. However, a relationship has not been defined, but we found that a good value to start with is the width of the mouth of the river. Once smoothed and aggregated (Figure 2), Step 1 of the tool has finished.

Figure 2

Step 4: Edit features in Arc Map

The user now must edit the shapefile (named “smoothed.shp”) in ArcMap. Often, in flat areas such as the Midwest, large tracts of upland is selected due to the flat topography (Figure 3), it is advisable to remove these large areas, and any other area which the user deems as an inappropriate surface to keep as a terrace or floodplain (i.e. roads, water surfaces, upland area, alluvial fans, etc.) Along with the removal of surface which the user is not interested in, we found that fights of terraces which should be mapped as individual surfaces are selected as one single surface.  The surface should be mapped as an individual surface but were connected by small “pig tail” portions of the polygon (Figure 4a). These segments can be remove by editing the polygon in ArcMap, using the the Cut Polygons tool (Editor Tool Box). The removal of these joining segments, allows the single polygon to be split into surfaces which will correctly represent the terraces on the landscape (Figure 4b). Here is where the set of aerial imagery, the DEM hillshade, and field observations will provide the best information when determining where to separate terraces which have been joined.

Figure 3

a b

Figure 4a and 4b

It might occur to the user to delete multiple files from Arc, which have been generated during Step1.  Do not erase any files as it will cause the tool to crash and produce undesirable results.  If using Arc 10.0 after the editing of the shapefile we found that due to the lock files (.loc) produced during the editing of the smoothed.shp file, it is advisable to close both ArcMap and ArcCatalog. This will delete the lock file, allowing Step 2 to continue uninterrupted. Otherwise, if using Arc 10.1, then simply remove the now edited ‘smoothed.shp’ from the ArcMap instance and move on.

Step 5: Open Step 2 of the tool and load parameters

Once the smoothed.shp file has been edited, and saved to the same directory, Step 2 of the tool can be started (Figure 5).  The areas of the now edited surfaces are re-calculated and the average elevation of the polygon surfaces calculated using the input DEM.  To calculate the average elevation above the river’s surface, the input stream shapefile is split based on the user defined reach length. Once split, the average elevation of each reach is calculated.   The terrace and floodplain features are then joined to the stream layer based on the proximity (channel width), and the average elevation of the stream is subtracted from the average elevation of the terrace/floodplain providing an average elevation of the surface above the river. The terraces/floodplains which are near the river (within the distance of the channel width) are mapped to a polygon steam layer, representing surfaces which are near the channel.

Figure 5

Step 6:  Outputs described and explained

The outputs from TerEx are: a local relief map, a stream line split into the reach length specified, a polygon shapefile representing the terrace and floodplain surfaces, and a polygon shapefile representing the stream with the relative elevation of the closest terrace to that section of the river.

The user may notice that in some instances no surface was mapped to the channel, this is because there was not a terrace/floodplain within the channel width.  This is likely because the floodplain near the channel was sufficiently rough to not be selected during the initial selection from the relief map (due to scroll bars and size).

The most important ability of this tool is the option to fine tune parameters used in selection.  We found in developing and when running the tool it works well to experiment on a small section of the watershed and then once the parameters are working well for the test area, to then apply the tool to the entire watershed.  This will remove the loss of time while waiting for the entire watershed to run, as well as having to make edits each time.

Step 7: How you can use the outputs

Create a map of the terraces.  In this map, a likely flight of terraces is visible

Figure 6

Figure 7

Compare the relative heights of bank elevations spatially; higher banks may be an area of a likely sediment source

Figure 8

Export table from stream_terraces.shp and open in excel to calculate the fraction of stream/river banks which are taller than a certain elevation (basically bank height relative to the channel (Figure 9).

Do this by first ranking the elevations (elev_abv) highest to lowest, then calculate the exceedance probability by the simple equation (m / (N+1)), where m is the rank, and N is the number of observations .

Figure 9

Even though we only present these minimal uses for the results from the TerEx tool, there are other uses. It would not be overly difficult to extract longitudinal profiles from the stream shapefile, or extract paleo-profiles from the terraces elevations.

If at any point you experience problems with running the script, or with getting reasonable results please feel free to contact Justin Stout at justin.stout@aggiemail.usu.edu or by phone +61 04 2747 2890.  I am very willing to help troubleshoot the problems and rectify any error which you may notice.